# Architectural and Engineering Specification

# AltoReach

# October 17, 2025

# Summary

The intent of this document is to specify the minimum criteria for the design, supply, and installation of AltoReach long-range 60GHz wireless radios.

Product: V band long-range radio that provides wireless point-to-point and point-to-multipoint connectivity based on the 802.11ad standard, with 2.5GbE Ethernet support.

AltoReach radios provide carrier-grade network connectivity for Ethernet applications such as high-definition megapixel cameras, multi-sensor cameras, and network video recorders (NVRs), as well as extending networks at fiber-quality speeds to provide Wi-Fi backhaul, smart city municipal building-top and street-level implementations, and connecting law enforcement locations.

Related requirements:

    - Features and functionality

      - Operation

      - Connectivity

      - Security

      - System management

    - Device Power

    - Device dimensions

    - Physical installation

    - Enclosure

    - Certifications

# Specification

  - The AltoReach AP90 and S150 radios shall include the following features and functionality:

    - The AP90 and S150 shall be able to function as an Access Point or Station.

      - When functioning in Access Point mode, the AltoReach radio shall support up to 32 connected Stations.

    - The AP90 shall provide 90° horizontal and +/-20° vertical coverage.

    - The S150 shall provide 4° horizontal and vertical coverage.

    - The AP90 and S150 shall operate in the unlicensed 57-71GHz band.

      - The AP90 and S150 shall provide 6 non-overlapping 2.16GHz channels within the 57-71GHz band.

        - Half-channel setting allows for 12 non-overlapping 1GHz channels.

      - Channel access shall be TDD.

    - The AP90 and S150 shall have 12 levels of Modulation and Coding Schemes from MCS-0 (BPSK) to MCS-12 (16QAM).

      - Half-channel setting has a maximum MCS of 9.

    - The AP90 and S150 shall have a capacity of up to 2+Gbps aggregate.

    - Ethernet port:

      - The AP90 and S150 shall have 1 RJ-45 2.5G BASE-T Ethernet port (with auto-negotiation speeds of 1Gbps/100Mbps).

      - The AP90 and S150 shall have IP67-rated ingress protection.

      - The Ethernet port shall be used to provide power to the device by using 802.3at Power over Ethernet (PoE).

    - The AP90 and S150 shall have transparent Ethernet bridging types including VLAN.

    - The AP90 and S150 shall utilize hardware-based AES-128 encryption for the 60GHz radio.

- The AP90 and S150 shall utilize hardware-based AES-256 encryption for the AES 256 for the Wi-Fi 7 radio.

    - The AP90 and S150 shall have an operating temperature range of -40° to +131°F 
(-40°C to +55°C).

  - System Management:

    - The AP90 and S150 shall provide a WebUI accessed from a desktop browser for configuration, management, and monitoring.

      - The AP90 and S150 shall also be optionally supported with AltoCommand, Altowav's enterprise management and monitoring UI.

      - The AP90 and S150 shall be able to be operated without AltoCommand.

    - The AP90 and S150 shall support the HTTPS protocol.

    - The AP90 and S150 shall provide a REST API for management and monitoring.

    - The AP90 and S150 shall provide the ability to save the unit configuration and upload it from local and remote connection.

    - The AP90 and S150 shall provide the ability to update configuration over the network (in-band).

    - The AP90 and S150 shall provide the ability to upgrade software over the network (in-band).

    - The AP90 and S150 shall default to using a static IP address.

      - The AP90 and S150 shall be able to be configured to use dynamic IP address assignment (DHCP).

    - The AP90 and S150 shall have an LED light to indicate device status.

      - The LED shall provide visual indication of the status of the device, including:

        - When the device is powering up.

        - When the device is waiting to form wireless connections.

        - When the device has formed wireless connections.

          - When a wireless connection is formed, the LED shall indicate the strength of the connection.

      - The AP90 and S150 shall also have Ethernet port LED lights, indicating power and link status.

  - Power:

    - The AP90 and S150 shall support Power over Ethernet (PoE) input.

    - The AP90 and S150 shall have a 25W maximum power consumption.

  - Dimensions:

    - The AP90 and S150 shall be 5 x 5 x 2 inches (130 x 130 x 55mm).

      - The S150 included antenna kit shall be 7 x 6 x 3 inches (300 x 300 x 80 mm).

    - The AP90 and S150 shall weigh 15 oz (425g).

      - The S150 included antenna kit shall be 34 oz (964g).

  - Physical installation

    - The AP90 and S150 shall provide an IP67-rated cable gland for installation of an Ethernet cable.

    - The AP90 and S150 shall provide wall-mounting and pole-mounting options without requiring additional brackets.

      - The AP90 and S150 shall have an optional pole mount bracket.

        - The pole mount bracket shall be attached to the pole by using band clamps.

        - The wall mount bracket shall enable azimuth adjustment.

Enclosure

    - Mechanical installation of the AP90 and S150 shall not require special tools.

    - The AP90 and S150 enclosure shall include all necessary link components:

      - Baseband.

      - Radio.

      - Scanning antenna.

        - The S150 shall include a provided external antenna kit.

    - The enclosure shall include the ability to be grounded.

The AP90 and S150 shall include the following certifications:

    - FCC ID: Z9W-TNA-303X

    - IC: 1468A-TNA303X