# Architectural and Engineering Specification

# AltoPlex C410 and C420

# October 17, 2025

# Summary

The intent of this document is to specify the minimum criteria for the design, supply, and installation of the AltoPlex C410 and C420 60GHz wireless radios.

Product: V band radios that function in tandem with AltoPlex D621 radios to provide wireless point-to-multipoint connectivity based on the 802.11ay standard, with 1GbE Ethernet support.

AltoPlex radios provide carrier-grade network connectivity for Ethernet applications such as high-definition megapixel cameras, multi-sensor cameras, and network video recorders (NVRs), as well as extending networks at fiber-quality speeds to provide Wi-Fi backhaul, smart city municipal building-top and street-level implementations, and connecting law enforcement locations.

Related requirements:

    - Features and functionality

      - Operation

      - Connectivity

      - Security

      - System management

    - Device Power

    - Device dimensions

    - Physical installation

    - Enclosure

    - Certifications

# Specification

  - The AltoPlex C410 and C420 shall include the following features and functionality:

    - The C410 and C420 shall function as client nodes (CNs).

    - The C410 and C420 shall support one link to one AltoPlex D621, when the D621 is operating in distribution node (DN) mode.

    - The C410 and C420 shall provide 90° horizontal and +/-20° vertical coverage.

    - The C410 and C420 shall operate in the unlicensed 57-66GHz band.

      - The C410 and C420 shall support 4 non-overlapping 2.16GHz channels within the 57-66GHz band.

      - Channel access shall be TDMA/TDD.

      - Wireless configuration, including the channel, shall be performed on the distribution node (D621) and propagated to the C410/C420 when the link is formed.

    - The C410 and C420 shall have 12 levels of Modulation and Coding Schemes from MCS-0 (BPSK) to MCS-12 (16QAM).

    - The C410 and C420 shall have a capacity of up to 2Gbps aggregate.

    - The C410 and C420 shall have less than 1ms latency.

    - Ethernet port:

      - The C410 and C420 shall have 1 RJ-45 1G BASE-T Ethernet port (with auto-negotiation to 100Mbps).

      - The C410 and C420 shall have IP67-rated ingress protection.

      - The C410 and C420's Ethernet port shall be used to provide power to the device by using 802.3at Power over Ethernet (PoE).

    - The C410 and C420 shall have transparent Ethernet bridging types including VLAN and VLAN stacking.

    - The C410 and C420 shall utilize hardware-based AES-128 encryption.

    - The C410 and C420 shall have an operating temperature range of -40° to +131°F 
(-40° to +55°C.)

  - System Management:

    - The C410 and C420 shall provide a WebUI accessed from a desktop browser for configuration, management, and monitoring.

      - The C410 and C420 shall also be optionally supported with AltoCommand, Altowav's enterprise management and monitoring UI.

      - The C410 and C420 shall be able to be operated without AltoCommand.

    - The C410 and C420 shall support various management protocols: HTTPS, SSH.

    - The C410 and C420 shall provide a REST API for management and monitoring.

    - The C410 and C420 shall provide the ability to save the unit configuration and upload it from local and remote connection.

      - Configuration backup and restore shall be supported via the REST API.

    - The C410 and C420 shall provide the ability to update configuration over the network (in-band).

    - The C410 and C420 shall provide the ability to upgrade software over the network (in-band).

    - The C410 and C420 shall default to using a static IP address.

      - The C410 and C420 shall be able to be configured to use dynamic IP address assignment (DHCP).

    - The C410 and C420 shall have a single green/red bi-color LED.

      - The LED shall provide visual indication of the status of the device, including:

        - When the device is waiting to form connections.

        - When the device has a wired connection and at least one wireless connection.

        - When the device is powering up.

        - When the device is in an error condition.

      - The C410 and C420 shall provide the ability to set the LED into "locate mode" which flashes a specific sequence to allow the device to be located.

      - The C410 and C420 shall provide the ability to disable the LED.

    - The C410 and C420 shall include an integrated Global Positioning System (GPS) receiver.

      - GPS shall provide location information for the C410/C420.

      - GPS support shall be enabled by default.

      - GPS support shall be able to be disabled from within the WebUI.

  - Power:

    - The C410 and C420 shall support Power over Ethernet (PoE) input.

      - The C420 shall have an 11W maximum power consumption.

      - The C410 shall have a 10W maximum power consumption.

  - Dimensions:

    - The C410 and C420 shall be 5.9 x 3.8 x 1.4 inches (150 x 95 x 35mm).

    - The C410 and C420 shall weigh 14 oz (400g).

  - Physical installation:

    - The C410 and C420 shall provide wall-mounting and pole-mounting options without requiring additional brackets.

    - The C410 and C420 shall have an optional wall mount bracket.

      - The C410 and C420 shall be attached to the wall mount bracket by using band clamps.

      - The wall mount bracket shall enable azimuth adjustment.

      - The wall mount bracket shall have an optional pole extension to support multiple devices and increased elevation.

    - The C410 and C420 shall have an optional tilting mount bracket.

      - The tilting mounting bracket shall be able to be mounted to a wall or flat surface by using screws.

      - The tilting mounting bracket shall be able to be mounted to a pipe or pole by using band clamps.

      - The tilting mounting bracket shall allow for tilting elevation adjustments from +60° to -45°.

    - The C410 and C420 shall provide an IP67-rated cable gland for installation of an Ethernet cable.

Enclosure

    - Mechanical installation of the C410 and C420 shall not require special tools.

    - The C410 and C420 enclosure shall include all necessary link components:

      - Baseband.

      - Radio.

      - Scanning antenna.

      - GPS antenna and module.

    - The enclosure shall include the ability to be grounded.

    - The enclosure shall be made of cast aluminum A338.

    - The enclosure shall have a 60-100 microns power coating of pantone PMS 427C paint.

    - The C410 and C420 front cover shall be made of PC/ABS thermoplastic.

    - The front cover shall include 0.5% HALS 770 UV light stabilizer.

The C410 and C420 shall include the following certifications:

    - Radio:

      - C410:

        - FCC ID: 2AMP5-46101

        - IC: 22992-46201

      - C420:

        - FCC ID: 2AMP5-46211

        - IC: 22992-46211

    - ESD: IEC EN 61000-4-2

    - EMC: IEC EN 61000-4-3