# Architectural and Engineering Specification

# Altowav K60

# February 27, 2025

# Summary

The intent of this document is to specify the minimum criteria for the design, supply, and installation of the Altowav K60 60GHz wireless radio.

Product: V band radio that provides wireless point-to-point and point-to-multipoint connectivity based on the 802.11ay standard, with 1GbE Ethernet support.

AltoPlex radios provide carrier-grade network connectivity for Ethernet applications such as high-definition megapixel cameras, multi-sensor cameras, and network video recorders (NVRs), as well as extending networks at fiber-quality speeds to provide Wi-Fi backhaul, smart city municipal building-top and street-level implementations, and connecting law enforcement locations.

Related requirements:

    - Features and functionality

      - Operation

      - Connectivity

      - Security

      - System management

    - Device Power

    - Device dimensions

    - Physical installation

    - Enclosure

    - Certifications

# Specification

  - The Altowav K60 shall include the following features and functionality:

    - The K60 shall be able to function as a Hub or a Remote.

    - When operating in Hub mode, the K60 shall support up to 8 connections to Remotes.

    - The K60 shall provide 90° horizontal and +/-20° vertical coverage.

    - The K60 shall operate in the unlicensed 57-66GHz band.

      - The K60 shall provide 3 non-overlapping 2.16GHz channels within the 57-66GHz band.

      - Channel access shall be TDD.

    - The K60 shall have 8 levels of Modulation and Coding Schemes from MCS-0 (BPSK) to MCS-8 (QPSK).

    - The K60 shall have a capacity of up to 1.8Gbps aggregate.

    - The K60 shall have less than 1ms latency.

    - Ethernet ports:

      - The K60 shall have 2 RJ-45 1G BASE-T Ethernet ports (with auto-negotiation speeds of 1Gbps/100Mbps).

        - One K60 Ethernet port shall be used to provide power to the device by using 802.3af/at/bt Power over Ethernet (PoE).

        - One K60 Ethernet port shall be used as active 802.3af/at PoE output up to 30W.

      - The K60 Ethernet ports shall have IP67-rated ingress protection (requires HARTING IP67 connectors).

    - The K60 shall have transparent Ethernet bridging types including VLAN and VLAN stacking.

    - The K60 shall utilize hardware-based AES-128 encryption.

    - The K60 shall have an operating temperature range of -40° to +131°F (-40° to +55°C.)

  - System Management:

    - The K60 shall provide a WebUI accessed from a desktop browser for configuration, management, and monitoring.

      - The K60 shall also be optionally supported with AltoCommand, Altowav's enterprise management and monitoring UI.

      - The K60 shall be able to be operated without AltoCommand.

    - The K60 shall support various management protocols: HTTPS, SSH.

    - The K60 shall provide a REST API for management and monitoring.

    - The K60 shall provide the ability to save the unit configuration and upload it from local and remote connection.

      - Configuration backup and restore shall be supported via the REST API.

    - The K60 shall provide the ability to update configuration over the network (in-band).

    - The K60 shall provide the ability to upgrade software over the network (in-band).

    - The K60 shall default to using a preconfigured, unique static IP address.

    - The K60 shall be able to be configured to use dynamic IP address assignment (DHCP).

    - The K60 shall have a single green/red bi-color LED.

      - The LED shall provide visual indication of the status of the device, including:

        - When the device is in Hub mode and is the device operating normally.

        - When the device is in Remote mode and is waiting to form a link to a hub.

        - When the device is in Remote mode and is connected to a hub.

        - When the device is powering up.

        - When the device is in an error condition.

      - The K60 shall provide the ability to set the LED into "locate mode" which flashes a specific sequence to allow the device to be located.

      - The K60 shall provide the ability to disable the LED.

  - Power:

    - The K60 shall have Power over Ethernet (PoE) support:

      - One K60 Ethernet port shall be used to provide power to the device by using 802.3af/at/bt Power over Ethernet (PoE).

      - One K60 Ethernet port shall be used as active 802.3af/at PoE output up to 30W.

    - The K60 shall have a 8W maximum power consumption, not including PoE output.

      - Including PoE output, the K60 shall have a total power consumption of up to 38W.

  - Dimensions:

    - The K60 shall be 6.2 x 3.9 x 1.9 inches (157 x 99 x 48 mm).

    - The K60 shall weigh 14 oz (400g)

  - Physical installation

    - The K60 shall provide a pole/wall mounting bracket which allows for adjusting the vertical tilt.

    - The K60 shall have an optional wall standoff bracket that allows for azimuth adjustment when mounting the device to a wall.

    - The K60 shall provide an IP67-rated cable gland for installation of an Ethernet cable.

Enclosure

    - Mechanical installation of the K60 shall not require special tools.

    - The K60 enclosure shall include all necessary link components:

      - Baseband.

      - Radio.

      - Scanning antenna.

    - The enclosure shall include the ability to be grounded.

The K60 shall include the following certifications:

    - Radio:

      - FCC ID: 2AMP5K60

      - IC: 22992-K60

    - ESD: IEC EN 61000-4-2

    - EMC: IEC EN 61000-4-3